# Rolling stock data for traction power simulation

Sample 6-car metro. Replace the values with the manufacturer's data. The model reads 'Parameter : value unit' lines, two-column parameter tables, curve tables with a Speed column, Word charts, and graph images (axes calibrated in the digitiser).

## 1. General data

| Parameter | Value | Unit |
| --- | --- | --- |
| Number of cars | 6 |  |
| Number of traction motors | 16 |  |
| Train length | 132 | m |
| Simulation mass (AW3, crush load) | 260 | t |
| Rotating mass allowance | 8 | % |
| Maximum operating speed | 80 | km/h |
| Starting acceleration | 1 | m/s² |
| Service braking rate | 1 | m/s² |
| Emergency braking rate | 1.3 | m/s² |
| Auxiliary power per car | 40 | kW per car |
| Nominal line voltage | 750 | V DC |
| Maximum regenerative braking power | 3500 | kW |

Running resistance (Davis): R = A + B·v + C·v² with A = 3.5, B = 0.03, C = 0.0007 (kN, v in km/h)

## 2. Tractive and electric braking effort (whole train, at wheel)

| Speed (km/h) | Tractive effort (kN) | Electric braking effort (kN) | Drive efficiency (%) |
| --- | --- | --- | --- |
| 0 | 290 | 0.0 | 75.0 |
| 10 | 290 | 290.0 | 82.0 |
| 20 | 290 | 290.0 | 86.0 |
| 30 | 290 | 290.0 | 88.0 |
| 36 | 290 | 290.0 | 88.0 |
| 40 | 261 | 275.7 | 88.0 |
| 50 | 208.8 | 240.0 | 88.0 |
| 60 | 174 | 210.0 | 88.0 |
| 70 | 127.8 | 180.0 | 88.0 |
| 80 | 97.9 | 155.0 | 88.0 |
| 90 | 77.3 | 140.0 | 88.0 |

## 3. Tractive effort graph

Graph image: X axis 0 - 100 km/h, Y axis 0 - 350 kN (plot frame edges). Use these values when calibrating the digitiser.